Assignment: Network Fundamentals and Building Networks

Section 1: Multiple Choice

1.What is the primary function of a router in a computer network?

Forwarding data packets between networks

2. What is the purpose of DHCP (Dynamic Host Configuration Protocol) in a computer network?

Dynamically assigning IP addresses to devices

3. Which network device operates at Layer 2 (Data Link Layer) of the OSI model and forwards data packets based on MAC addresses?

Switch

4. Which network topology connects all devices in a linear fashion, with each device connected to a central cable or backbone?

Bus

Section 2: True or False

5. True or False: A VLAN (Virtual Local Area Network) allows network administrators to logically segment a single physical network into multiple virtual networks, each with its own broadcast domain

True

6. True or False: TCP (Transmission Control Protocol) is a connectionless protocol that provides reliable, ordered, and error-checked delivery of data packets over a network.

False

7. True or False: A firewall is a hardware or software-based security system that monitors and controls incoming and outgoing network traffic based on predetermined security rules.

True

Section 3: Short Answer

8. Describe the steps involved in setting up a wireless network for a small office or home office (SOHO) environment.

1. Planning and Equipment Selection.

A. Assess Needs

- Determine Requirements: How many devices will connect (laptops, phones, printers, smart devices)? What are the primary uses (e.g., video conferencing, large file transfers, simple browsing)? This dictates the required speed and coverage.

- Layout and Coverage: Note the location of the modem and where coverage is most critical. Identify potential obstacles like thick walls or metal objects.

B. Choose Equipment

- Modem: This connects your network to your Internet Service Provider (ISP). In many cases, the ISP provides this.

- Router (Wireless Router): This is the core of your network.

  - Standards: Look for modern standards like Wi-Fi 6 (802.11ax) or newer for the best performance and security.

  - Range/Type: For larger spaces, consider a Mesh Wi-Fi System to ensure uniform coverage. For smaller areas, a single, powerful router is sufficient.

- Ethernet Cables: Use Cat 5e or Cat 6 for connections between the modem and router, and any wired devices.

2. Physical Setup and Connections

Placement of the Router: Place the router in a central, open location, away from obstructions and potential interference sources (like microwave ovens or cordless phones). Positioning it high up is often beneficial.

Connect the Modem:

- Plug the modem into the wall jack (coaxial, telephone, or fiber, depending on your ISP).

- Connect the modem's power.

Connect the Router:

- Connect one end of an Ethernet cable to the WAN (or Internet) port on your wireless router.

- Connect the other end to the Ethernet port on your modem.

- Connect the router's power.

Power On: Turn on the modem first and wait for it to fully boot up (usually indicated by stable status lights). Then, turn on the router.

3. Configuration via Computer

Access Router Interface: Open a web browser and enter the router's default IP address (e.g., $192.168.1.1$, $192.168.0.1$, or a specific web address like routerlogin.net). This information is usually found in the router's manual or on a sticker on the device.

Log In: Enter the default administrative username and password (e.g., admin/admin or admin/password). IMMEDIATELY change this default login to a strong, unique password to prevent unauthorized access.

Run Setup Wizard: Most modern routers have a setup wizard that will guide you through the next steps.

4. Configuring Wireless Settings and Security

A. Network Name (SSID)

- Change the Default SSID: Set a unique and descriptive name for your network (e.g., "OfficeNet"). Do not use personal information.

B. Security Protocol

- Select Strong Encryption: Always choose WPA3 if your devices support it, or WPA2-PSK (AES) as the minimum standard. Avoid WEP and WPA (TKIP), as they are insecure.

C. Wireless Password

- Set a Strong Key: Create a long, complex passphrase (minimum of 12 characters, including upper/lowercase letters, numbers, and symbols). This is the key people will use to connect to your Wi-Fi.

D. Other Important Settings

- Update Firmware: Check for and install the latest firmware for your router. This is vital for security patches and performance improvements.

- Enable Firewall: Ensure the router's built-in firewall is enabled.

- Set up a Guest Network (Optional but Recommended): Enable a separate guest network for visitors. This keeps them off your main private network, enhancing security.

5. Testing and Troubleshooting

Connect Devices: Disconnect the configuration computer from the Ethernet cable and try to connect it and other devices wirelessly using the new SSID and password.

Test Speed and Range:

- Run an Internet speed test near the router to verify your connection speed.

- Move to the farthest areas of the SOHO and test the speed and signal strength to ensure adequate coverage.

Troubleshoot Weak Signals: If you have weak spots, you may need to adjust the router's antenna positions, move the router, or consider adding a Wi-Fi repeater or access point.

Section 4: Practical Application

9. Demonstrate how to configure a router for Internet access using     DHCP (Dynamic Host Configuration Protocol).

1. Physical Connection and Accessing the Router

Connect the Modem to the Router: Connect an Ethernet cable from your ISP's modem (or optical network terminal/ONT) to the WAN (Wide Area Network) or Internet port on your wireless router.

Connect for Configuration: Connect a computer directly to one of the LAN (Local Area Network) ports on the router using a second Ethernet cable.

Access the Interface:

- Open a web browser on the connected computer.

- Enter the router's default IP address (the Default Gateway). This is usually printed on a sticker on the router, but common addresses include 192.168.1.1, 192.168.0.1, or 10.0.0.1.

- Log in using the router's administrative username and password (which you should change immediately if you haven't already

Configuration Command:

Router> enable

Router# configure terminal

Router(config)# interface GigabitEthernet0/0

Router(config-if)# ip address dhcp

Router(config-if)# no shutdown

Router(config-if)# exit

2. WAN (Internet) Configuration - The DHCP Client Role

The router acts as a DHCP Client to your ISP, requesting a public IP address.

| Step | Action | Description |
| --- | --- | --- |
| 1. | Navigate to Internet/WAN Settings | Find the section typically labeled Internet, WAN, or Network Settings. |
| 2. | Select Connection Type | Under WAN Connection Type or similar, select Dynamic IP or DHCP. |
| 3. | Review/Apply | The router will typically not require any further input. It will send a DHCP Discover request to the ISP's network and automatically receive its public IP Address, Subnet Mask, Default Gateway, and DNS Servers. |
| 4. | Save Changes | Click Save or Apply and wait for the router to connect to the Internet. You should see a "Connected" status. |

Configuration Command:

Router# show ip interface brief

3. LAN (Local) Configuration - The DHCP Server Role

A. Enabling the DHCP Server

- Navigate to LAN/Network Settings: Find the section typically labelled LAN Settings, Network Setup, or DHCP Server.

- Enable DHCP: Ensure the DHCP Server option is set to Enabled (this is usually the default).

B. Configuring the IP Address Pool (Scope)

This defines the range of private IP addresses the router can assign to devices.

| Parameter | Example Value | Description |
| --- | --- | --- |
| Router's IP Address (Gateway) | 192.168.1.1 | This is the permanent address of the router itself. All local traffic passes through this. |
| Subnet Mask | 255.255.255.0 | The standard mask for a small network, defining the size of the local network. |
| Starting IP Address | 192.168.1.100 | The first IP the DHCP server will assign. |
| Ending IP Address | 192.168.1.200 | The last IP the DHCP server will assign. |
| IP Pool Range | 192.168.1.100 192.168.1.200 | This leaves addresses (like .2 to .99) free for manually configuring devices that require a Static IP. |

C. Setting the Lease Time

- Lease Time: Set the duration (e.g., 1 hour, 24 hours, 7 days) that a device is allowed to hold onto its assigned IP address before it must renew the lease. For a SOHO, 24 hours is a common and stable setting.

D. Applying Changes

- Save and Reboot: Save the changes. The router may require a reboot for the new DHCP settings to take full effect.

Configuration Command:

Router(config)# interface GigabitEthernet0/1

Router(config-if)# ip address 192.168.1.1 255.255.255.0

Router(config-if)# no shutdown

Router(config-if)# exit

Configure DHCP Server for LAN Clients (Optional but Common)

Router(config)# ip dhcp excluded-address 192.168.1.1 192.168.1.10

Create DHCP Pool:

Router(config)# ip dhcp pool LAN_POOL

Router(dhcp-config)# network 192.168.1.0 255.255.255.0

Router(dhcp-config)# default-router 192.168.1.1

Router(dhcp-config)# dns-server 8.8.8.8 8.8.4.4

Router(dhcp-config)# exit

Configure NAT for Internet Access (Required):

Identify Inside and Outside Interfaces:

Router(config)# interface GigabitEthernet0/1

Router(config-if)# ip nat inside

Router(config-if)# exit

Router(config)# interface GigabitEthernet0/0

Router(config-if)# ip nat outside

Router(config-if)# exit

Configure NAT Overload (PAT)

Router(config)# access-list 1 permit 192.168.1.0 0.0.0.255

Router(config)# ip nat inside source list 1 interface GigabitEthernet0/0 overload

- Verification

After the router reboots, all connected devices (laptops, smartphones, etc.) should:

- Request an IP: Send a DHCP Request.

- Receive an IP: Be automatically assigned an IP address, Subnet Mask, Default Gateway (the router's IP), and DNS server address from the router's newly configured DHCP pool.

To verify, you can check the network settings on a client device. The IPv4 configuration should be set to Obtain an IP address automatically (i.e., using DHCP).

Verify Internet Connectivity

Check DHCP Lease:

Router# show dhcp lease

Check NAT Translations:

Router# show ip nat translations

Test Connectivity

Router# ping 8.8.8.8

Section 5: Essay:

10. Discuss the importance of network documentation in the context of building and managing networks.

1. Efficient Troubleshooting and Reduced Downtime:

Rapid Incident Response: Detailed records (like IP addresses, port assignments, and configuration backups) allow engineers to quickly isolate the root cause of an issue without guesswork.

Reduced MTTR (Mean Time to Repair): By having immediate access to Network Topology Diagrams, engineers can visualize the affected devices and connections, significantly speeding up the time it takes to fix an outage

Consistent Resolution: Documentation establishes standard operating procedures (SOPs) for common problems, ensuring all staff follow the same proven steps, reducing human error.

2. Knowledge Transfer and Business Continuity

Eliminating Knowledge Silos: When key IT staff members leave, their institutional knowledge about the network leaves with them. Up-to-date documentation preserves all critical design decisions, access credentials, and historical changes

Smoother Onboarding: New employees can be brought up to speed much faster by studying the network's detailed blueprint and standard procedures.

Disaster Recovery: A detailed Network Recovery Plan is impossible without documentation that specifies backup schedules, critical system dependencies, and the step-by-step process for restoring the network to a pre-failure state.

3. Change Management and Planning

Controlled Upgrades: Before implementing a change (e.g., upgrading a router or adding a new server), documentation allows engineers to understand all dependencies, preventing unintended service disruptions

Change Logs: Maintaining a detailed history of modifications—including what was changed, when, why, and by whom—is vital for auditing and troubleshooting post-change issues

Capacity Planning: Accurate documentation, especially the IP addressing scheme and hardware inventory, helps management forecast resource needs, justify budget requests for new equipment, and identify underutilized assets

4. Security and Compliance

Audit Preparedness: Many industry regulations (e.g., HIPAA, GDPR, PCI DSS) require detailed documentation of network configurations, security policies, and access controls as proof of compliance.

Vulnerability Identification: Documenting firewall rules, VPN configurations, and the version numbers of all hardware and software helps security teams proactively identify and patch devices with outdated or vulnerable firmware

Access Control: A documented record of user accounts, administrative passwords (stored securely), and access policies ensures only authorized personnel can make changes to the network